CONBEL

Este es un sistema básico del control del belén por ordenador, simplemente lo que hace es conectar el programador de fases, y reproducir la música, al mismo tiempo que desconecta la ilumunación exterior al belén.

Como se ve es un sistema bastante sencillo.

Programación de CONBEL.

Programar el CONBEL, es sencillo en sí, sólo necesitamos en principio tres instrucciones BASIC: INP, OUT y SLEEP.

Función INP: genera un byte leido del puerto (hardware) de Entrada y Salida (E/S).

INP(puerto%)

Donde puerto%  =  Un numero entre 0 y 65,535 que identifica el puerto.

Función OUT: envia un byte a un puerto (hardware) de E/S.

OUT puerto%, datos%

Donde datos% =   Una expresion numerica entre 0 y 255 que es enviada

al puerto.

Ejemplo:

A = INP(&H378)

Rem Lee desde el puerto paralelo

OUT &H378, 255

Rem pone todos los bits a 1

Instrucción SLEEP

Suspende temporalmente la ejecucion del programa.

SLEEP [segundos]

Donde segundos =   El numero de segundos durante los que se suspende

la ejecucion del programa.

Si segundos es 0  o se omite, el programa es suspendido hasta que

se presione una tecla o hasta que ocurra un evento interceptado.

Ejemplo:

CLS

Rem limpia la pantalla

Out &h378,128

Rem escribe el valor 128 en elpuerto paralelo

A = INP(&H378)

Rem Lee desde el puerto paralelo

PRINT "A"

Rem Escribe el valor leido en pantalla

SLEEP 10

Rem Espera 10 segundos

CLS

Rem limpia la pantalla

El proceso general, en su forma más simple, es tan sencillo como:

- Escribir en el puerto la combinación elegida.

- Esperar un tiempo.

- Volver a escribir en el puerto.

- Nueva pausa hasta la siguiente escritura.

Así según las fases que deseemos tener.

Esto nos daría un ciclo, por lo tanto, deberemos repetir este código, tantas veces como ciclos deseemos repetir.

Obviamente si queremos darle al programa un aspecto agradable, y reproducir música, el BASIC se nos queda corto, deberemos usar al menos Visual Basic.

PROGRAMA DE EJEMPLO:

CONBEL para QBASIC en MSDOS.

REM * Control de Belen *

REM * Asignaci¢n de posiciones de Memoria *

P1A = &H1B0

P1B = &H1B1

P1C = &H1B2

IC82551 = &H1B3

P2A = &H1B4

P2B = &H1B5

P2C = &H1B6

IC82552 = &H1B7

CONT0 = &H1B8

CONT1 = &H1B9

CONT2 = &H1BA

IC8253 = &H1BB

REM * Configuraci¢n Inicial *

CONFIG8253 = 0

CONFIG82551 = 128

CONFIG82552 = 128

CONFIGCONT0 = 0

CONFIGCONT1 = 0

CONFIGCONT2 = 0

S1A = 0

S1B = 0

S1C = 0

S2A = 0

S2B = 0

S2C = 0

REM * PANTALLA PRINCIPAL*

CLS

COLOR 14, 0

LOCATE 1, 1:  PRINT "ÉÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍ»"

LOCATE 2, 1:  PRINT "º                                                                              º"

LOCATE 3, 1:  PRINT "ÌÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍ¹"

FOR a = 4 TO 22

LOCATE a, 1:  PRINT "º                                                                              º"

NEXT a

FOR a = 2 TO 79

LOCATE 5, a:  PRINT "Ä"

NEXT a

FOR a = 2 TO 43

LOCATE 15, a:  PRINT "Ä"

NEXT a

FOR a = 4 TO 22

LOCATE a, 20:  PRINT "³"

NEXT a

FOR a = 4 TO 22

LOCATE a, 43:  PRINT "º"

NEXT a

FOR a = 4 TO 7

LOCATE a, 62:  PRINT "³"

NEXT a

FOR a = 2 TO 42

LOCATE 13, a:  PRINT "Í"

NEXT a

FOR a = 44 TO 79

LOCATE 7, a:  PRINT "Í"

NEXT a

FOR a = 44 TO 79

LOCATE 9, a:  PRINT "Ä"

NEXT a

LOCATE 23, 1: PRINT "ÈÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍÍ¼"

COLOR 28, 0

LOCATE 2, 25: PRINT "***   ESTADO DE LOS PUERTOS   ***"

COLOR 14, 0

LOCATE 4, 4: PRINT "CONFIG. ICs"

LOCATE 4, 25: PRINT "CONTADORES 8253"

LOCATE 14, 4: PRINT "PUERTOS 82551"

LOCATE 14, 25: PRINT "PUERTOS 82552"

LOCATE 4, 45:  PRINT "DIA:"

LOCATE 4, 64:  PRINT "HORA:"

LOCATE 6, 46:  PRINT "CICLO:"

LOCATE 6, 64:  PRINT "FASE:"

LOCATE 8, 52: PRINT "ESTADO DE LAS SALIDAS"

COLOR 4, 0

LOCATE 10, 51: PRINT "S8 S7 S6 S5 S4 S3 S2 S1"

LOCATE 12, 45: PRINT "P1A"

LOCATE 14, 45: PRINT "P1B"

LOCATE 16, 45: PRINT "P1C"

LOCATE 18, 45: PRINT "P2A"

LOCATE 20, 45: PRINT "P2B"

LOCATE 22, 45: PRINT "P2C"

COLOR 11, 0

LOCATE 6, 4: PRINT "IC8253="

LOCATE 8, 4: PRINT "IC82551="

LOCATE 10, 4: PRINT "IC82552="

LOCATE 6, 25: PRINT "CONT0="

LOCATE 8, 25: PRINT "CONT1="

LOCATE 10, 25: PRINT "CONT2="

LOCATE 16, 4: PRINT "P1A="

LOCATE 18, 4: PRINT "P1B="

LOCATE 20, 4: PRINT "P1C="

LOCATE 16, 25: PRINT "P2A="

LOCATE 18, 25: PRINT "P2B="

LOCATE 20, 25: PRINT "P2C="

REM * Estado Inicial *

FASE$ = "Inicio"

GOSUB Inicio

GOSUB ESCRIBEDATOS

GOSUB LEEDATOS

SLEEP 1

FOR CICLO = 1 TO 5

REM * Ciclo de 4 Fases *

Amanecer:

FASE$ = "Amanecer "

S1A = 15

S1B = 0

S1C = 15

S2A = 0

S2B = 15

S2C = 0

GOSUB ESCRIBEDATOS

GOSUB LEEDATOS

SLEEP 1

Dia:

FASE$ = "Dia      "

S1A = 240

S1B = 0

S1C = 240

S2A = 0

S2B = 240

S2C = 0

GOSUB ESCRIBEDATOS

GOSUB LEEDATOS

SLEEP 1

Tarde:

FASE$ = "Tarde    "

S1A = 0

S1B = 15

S1C = 0

S2A = 15

S2B = 0

S2C = 15

GOSUB ESCRIBEDATOS

GOSUB LEEDATOS

SLEEP 1

Noche:

FASE$ = "Noche    "

S1A = 0

S1B = 240

S1C = 0

S2A = 240

S2B = 0

S2C = 240

GOSUB ESCRIBEDATOS

GOSUB LEEDATOS

SLEEP 1

NEXT CICLO

CICLO = 0

FASE$ = "Final"

S1A = 0

S1B = 0

S1C = 0

S2A = 0

S2B = 0

S2C = 0

GOSUB ESCRIBEDATOS

GOSUB LEEDATOS

SLEEP 10

SYSTEM

REM * Configuraci¢n Inicial *

Inicio:

OUT IC8253, CONFIG8253

OUT IC82551, CONFIG82551

OUT IC82552, CONFIG82552

OUT CONT0, CONFIGCONT0

OUT CONT1, CONFIGCONT1

OUT CONT2, CONFIGCONT2

RETURN

REM * Subprograma de Escritura de datos

ESCRIBEDATOS:

OUT P1A, S1A

OUT P1B, S1B

OUT P1C, S1C

OUT P2A, S2A

OUT P2B, S2B

OUT P2C, S2C

RETURN

REM * Subprograma de Presentaci¢n de Estado *

LEEDATOS:

COLOR 7, 0

LOCATE 4, 51:  PRINT DATE$

LOCATE 4, 70:  PRINT TIME$

LOCATE 6, 52: PRINT CICLO

LOCATE 6, 70: PRINT FASE$

LOCATE 6, 13: PRINT "    ": LOCATE 6, 13: PRINT CONFIG8253

LOCATE 8, 13: PRINT "    ": LOCATE 8, 13: PRINT CONFIG82551

LOCATE 10, 13: PRINT "    ": LOCATE 10, 13: PRINT CONFIG82552

LOCATE 6, 32: PRINT "    ": LOCATE 6, 32: PRINT CONFIGCONT0

LOCATE 8, 32: PRINT "    ": LOCATE 8, 32: PRINT CONFIGCONT1

LOCATE 10, 32: PRINT "    ": LOCATE 10, 32: PRINT CONFIGCONT2

LOCATE 16, 9: PRINT "    ": LOCATE 16, 9: PRINT S1A

n = S1A

GOSUB BINARIO

LOCATE 12, 51

PRINT R7; R6; R5; R4; R3; R2; R1; R0

LOCATE 18, 9: PRINT "    ": LOCATE 18, 9: PRINT S1B

n = S1B

GOSUB BINARIO

LOCATE 14, 51

PRINT R7; R6; R5; R4; R3; R2; R1; R0

LOCATE 20, 9: PRINT "    ": LOCATE 20, 9: PRINT S1C

n = S1C

GOSUB BINARIO

LOCATE 16, 51

PRINT R7; R6; R5; R4; R3; R2; R1; R0

LOCATE 16, 30: PRINT "    ": LOCATE 16, 30: PRINT S2A

n = S2A

GOSUB BINARIO

LOCATE 18, 51

PRINT R7; R6; R5; R4; R3; R2; R1; R0

LOCATE 18, 30: PRINT "    ": LOCATE 18, 30: PRINT S2B

n = S2B

GOSUB BINARIO

LOCATE 20, 51

PRINT R7; R6; R5; R4; R3; R2; R1; R0

LOCATE 20, 30: PRINT "    ": LOCATE 20, 30: PRINT S2C

n = S2C

GOSUB BINARIO

LOCATE 22, 51

PRINT R7; R6; R5; R4; R3; R2; R1; R0

RETURN

REM * Conversi¢n de un n£mero en binario

BINARIO:

n1 = INT(n / 2)

n2 = INT(n1 / 2)

n3 = INT(n2 / 2)

n4 = INT(n3 / 2)

n5 = INT(n4 / 2)

n6 = INT(n5 / 2)

n7 = INT(n6 / 2)

n8 = INT(n7 / 2)

R0 = n - (n1 * 2)

R1 = n1 - (n2 * 2)

R2 = n2 - (n3 * 2)

R3 = n3 - (n4 * 2)

R4 = n4 - (n5 * 2)

R5 = n5 - (n6 * 2)

R6 = n6 - (n7 * 2)

R7 = n7 - (n8 * 2)

RETURN

Este programa, se puede cargar y ejecutar fácilmente en cualquier editor de QBASIC.